OCTG Hydrostatic Tester — Engineering Master v1.1

Canonical Source • PRJ-HYD-01 • Effective 24 Aug 2026

| Field | Value |
| --- | --- |
| Source ID | PRJ-HYD-01-ENG |
| Version | 1.1 |
| Status | CANONICAL |
| Authority scope | Current buyer-side engineering basis for supplier qualification, RFQ, contract review and FAT planning |
| Supersedes | Hydrostatic_Tester_Engineering_Master_v1.0_2026-08-23 |
| Controlling basis | GSY180/120 Final Technical Specification Rev.1.2 dated 22 Aug 2026 + later NEXUS reconciliation |

# Confirmed / Governing Requirements

Equipment: hydrostatic testing machine, nominal reference GSY180/120 or technically equivalent.

Buyer base OD: 89–180 mm.

Buyer base wall thickness: 6–20 mm.

Buyer base pipe length: 9–12 m. Earlier 6–12 m / 5–12 mm values are superseded for this Hydrotester package.

Maximum hydrostatic test pressure capability: 120 MPa (1,200 bar). Actual operating test pressure varies by geometry/duty; the vendor must submit a signed pressure-versus-OD/ID/WT/length/end-condition capability matrix.

Typical pressure hold time basis: 5–10 s.

Buyer production-capacity basis: 60 pipes/hour. FAT geometry, pressure and hold time used to prove this output must be contractually defined.

Known supplier reference for Ø168.3 mm: 50 pipes/hour at 120 MPa and 60 pipes/hour at 70 MPa; this is supplier evidence, not automatic acceptance of the 60 pipes/hour buyer basis at 120 MPa.

Pressure measurement/acceptance must be reduced to one unambiguous contractual pass/fail criterion with traceable calibration.

Qualification requires signed structural calculations/FEA for the worst contractual pressure and pipe-ID load envelope, including frame and test heads.

FAT/TPI/ITP, welding/NDE scope, sealing/end-condition capability, data recording/export, safety interlocks and no-substitution BOM controls must be explicitly closed before manufacturing release where applicable.

# Open / Hold Points

Final agreed OD × WT/ID × length × pressure × hold-time × guaranteed sustained output matrix.

Whether 60 pipes/hour is contractually achievable at each critical 120 MPa duty point; current Ø168.3 mm reference is only 50 pipes/hour at 120 MPa.

Final pressure accuracy/deviation/stability pass/fail rule.

End-condition exclusions for plain, upset, threaded and coupled pipe and any tooling/cycle penalties.

Final seal-life contractual basis and approved seal/packing vendor.

Final utilities, layout/interface, spare parts, training, installation, SAT and warranty terms.

# Supplier Evidence Status

Boyu standard public GSY180 variants around 70 MPa do not prove the required 120 MPa duty without documented custom capability.

GH Petro / HPG-180 and Marley proposals remain evidence/benchmarks until they close the signed capability matrix, throughput and acceptance hold points.

KARAT / Yaxing SYJ-I A-219 brochure is a high-value candidate because nominal geometry/pressure can reach OD 89–219 mm and up to 150 MPa, but brochure evidence does not establish cycle time or 60 pipes/hour.